## Literature Review Outline

### 1. Global Climate Change and Industrial Carbon Capture

- 1.1 The Need for Carbon Capture and Storage (CCS): Overview of industrial fossil-fuel emissions, global reduction targets, and post-combustion capture as a crucial short-to-medium-term solution.

- 1.2 Commercial Benchmark — Amine Scrubbing: How liquid amine systems work, their operational advantages, and their key limitations (high regeneration energy penalty, equipment corrosion, and toxic breakdown byproducts).

- 1.3 Shift Toward Solid Adsorbents: Brief comparison of solid materials (zeolites, activated carbons, silica, and MOFs) as lower-energy alternatives to liquid solvents.

### 2. Metal-Organic Frameworks (MOFs) for Gas Separation

- 2.1 Fundamentals of MOF Chemistry: Metal clusters (nodes) linked by organic ligands; key properties including ultra-high surface area, tunable pore size, and customizable surface chemistry.

- 2.2 $\text{CO}_2$ Capture Mechanisms: Difference between physical adsorption (physisorption) and chemical bonding (chemisorption); role of open metal sites (OMS) and organic functional groups.

- 2.3 Promising MOF Families: Performance highlights of benchmark frameworks like HKUST-1 (copper-based) and UiO/Zr-based series (known for structural stability).

### 3. Key Technical Challenges of Pure MOFs

- 3.1 Water Vapor Competition & Instability: How moisture in industrial flue gas ($5\text{--}15\%\text{ }\text{H}_2\text{O}$) competes with $\text{CO}_2$ for adsorption sites, causing structural degradation or severe capacity drops.

- 3.2 The Energy vs. Selectivity Trade-off: Why chemical functionalization (e.g., amine grafting) increases selectivity but leads to high heat of adsorption ($\Delta H_{\text{ads}} > 60\text{ kJ/mol}$), negating the low-energy advantage.

- 3.3 Powder vs. Pellet Dilemma: Operational issues with fine MOF microcrystalline powders in packed-bed reactors (high pressure drop, channeling) and the problem of binder-induced pore clogging during pelletization.

### 4. Strategies for MOF Composite & Hybrid Systems

- 4.1 Surface-Modified Inorganic Supports: Using porous alumina or silica as mechanical substrates; role of functional anchoring molecules like $p$-aminobenzoic acid (PABA) to control nucleation and dispersion.

- 4.2 Carbon-Based Matrices & Biochar: Integrating MOFs with biochar or activated carbon to introduce hydrophobicity, improve moisture resistance, and lower overall material costs using biomass waste.

- 4.3 In-Situ Synthesis Techniques: Methods for growing MOF crystals directly on support surfaces to prevent pore blockage and ensure strong interfacial bonding.

### 5. Thermodynamics and Dynamic Capture Evaluation

- 5.1 Isosteric Heat of Adsorption ($\Delta H_{\text{ads}}$): Importance of keeping binding energy in the optimal $30\text{--}45\text{ kJ/mol}$ window for energy-efficient thermal or vacuum regeneration.

- 5.2 Dynamic Breakthrough Performance: Testing materials under realistic mixed-gas flows ($\text{CO}_2/\text{N}_2/\text{H}_2\text{O}$) rather than relying solely on pure single-component equilibrium isotherms.

- 5.3 Cyclic Durability & Regeneration: Review of temperature-swing adsorption (TSA) and pressure-vacuum swing adsorption (VSA) efficiency over multiple cycles.

### 6. Summary and Identification of Research Gap

- 6.1 Critical Synthesis of Literature: Recapping how existing studies focus either on high-capacity pure MOFs (water-sensitive) or binder-formed pellets (reduced capacity).

- 6.2 The Unresolved Gap: The need for hybrid MOF composites (specifically HKUST-1 or Zr-MOFs on functionalized alumina/biochar) that combine water tolerance, low regeneration enthalpy, and structural durability for scalable carbon capture.